WESTERN PROVINCE

RUSIZI DISTRICT

GIHEKE TECHNICAL SECONDARY SCHOOL

TERM I, WEEKEND TEST

ACADEMIC YEAR: 2026-2027

SECTOR: ICT

TRADE: CSA

TVET LEVEL: 5

MODULE TITLE: COMPUTER DEPLOYMENT AND MAINTENANCE

DATE: 20/09/2026, Duration:3HOURS

MARKS: …………/100

INSTRUCTIONS TO CANDIDATES:

This paper is composed of Three, Sections (A, B and C)

Follow the instructions given below, and answer the indicated

Section A: Attempt all Questions (14) questions/55 Marks

Section B: Attempt any (3) three questions/30 Marks

Section C: Attempt only one (1) question/15 Marks

SECTION A: Attempt All questions /55 Marks

| Q1 | Which of the following actions should a developer take when handling sensitive user data in an application? A. Store all sensitive data in plain text for easier access B. Use strong encryption methods to protect sensitive data C. Store sensitive data only in session cookies D. Allow users to access all their data without authentication | 2marks |
| --- | --- | --- |
| Q2 | Which of the following actions is most effective when applying anti malware protection to an organization’s network? A. Installing anti-malware software only on company servers B. Regularly updating the anti-malware software and signatures across all devices C. Relying solely on a firewall for malware prevention D. Running anti-malware scans only when malware is suspected | 2marks |
| Q3 | Read Carefully the following statement related to the e-waste effects on environment and human health and answer by TRUE if the statement is correct or FALSE if the statement is wrong. A. Burning e-waste can release toxic chemicals into the air, causing serious health and environmental issues. B. Throwing old electronic devices into regular trash bins is an environmentally safe way to dispose of them. C. Exposure to toxic substances in e-waste can cause only respiratory problems in humans. D. E-waste contains only non-toxic substances that are safe for humans and the environment. | 4marks |
| Q4 | Write the letter corresponding to their right answer in the first column by matching related hardware fault in the second column with their appropriate description in the third column: Answers; Hardware Faults; Likely Cause. 1: ...........................; 1.Computer freezes randomly; A. Insufficient power supply. 2:............................; 2.Blue Screen of Death (BSOD); B. Faulty RAM module. 3:.............................; 3.Slow boot times; C. Hard drive nearing failure. 4:.............................; 4.Overheating; D. Driver conflict or outdated driver. 5:............................; 5.Intermittent Wi Fi connection; E. Overclocked CPU or insufficient cooling. | 5marks |
| Q5 | Read Carefully the following statement and answer by TRUE if the statement is correct or FALSE if the statement is wrong A. Installing every available software update immediately is always the best approach. B. Regularly backing up your data is a crucial part of preventive maintenance. C. Antivirus software alone is sufficient protection against all online threats. | 3marks |
| Q6 | During a computer lab practical session, learners are asked to identify common hardware faults and recognize key internal computer components while troubleshooting a desktop pc that fails to boot properly. Based on the situation state whether the following statements are True or False - A RAM failure can cause a computer to fail to start or display errors during operations. - A PSU failure can prevent the computer from powering on. - The motherboard, CPU, RAM and PSU are all internal hardware components of a computer - External peripherals such as keyboard and monitors are considered internal hardware components | 4marks |
| Q7 | A user report that their printer is not working correctly. The technician decides to upgrade the printer’s device driver to fix the issues. You are asked to apply the correct steps to update a device driver, and answer the following questions. - Which of the following is the correct sequences to update a device driver on windows operating system? - Open device manager select the device update driver restart the pc - Restart the pc open device manager uninstall device install windows update. - Open control panel delete all drivers restart the pc install new drivers. - Open task manager select device end process restart the pc - Before updating the printer driver what should the technician do first to ensure a safe update process? - Uninstall all installed printer applications immediately - Confirm the correct printer model and driver version required. - Disconnect all USB Devices from computer system - Disable the operating system firewall permanently. | 4marks |
| Q8 | A technician finds a damaged lithium-ion battery while cleaning old laptop in the workshop. The battery is swollen and potentially hazardous. You are asked to apply the correct safe disposal method. Based on the situation, answer the following multiple choice questions. - What is the correct way to safely dispose of a damaged battery? - Place it in regular trash bin - Place it in a labeled hazardous e-waste container - Throw it into a recycling bin for plastic - Burn it to remove chemical waste. - Before handling the swollen lithium-ion battery what is the first safety action the technician should take - Disconnect the power and wear appropriate protective equipment - Puncture the battery to release trapped internal gases - Place the battery in water to reduce overheating risk - Shake the battery to check internal chemical stability. |  |
| Q9 | In school, laboratory, a computer is frequently shutting down unexpectedly due to overheating issue. During a class session the IT technician asks learners to identify the common cause of CPU or system overheating. Which of the following are common cause of computer overheating? (select all possible cause) - Dust building in fans and vents - Poor airflow inside the case - Degraded thermal paste on the CPU - Installing the additional RAM module |  |
| Q10 |  |  |
| Q11 |  |  |
| Q12 |  |  |
| Q13 |  |  |
| Q14 |  |  |
| Q15 |  |  |
| Q16 |  |  |
| Q17 |  |  |

Section B

| Q18 | Why is it important to follow an organized sequence when assembling a computer, and how does this impact troubleshooting? at least 5 key points. | 10 marks |
| --- | --- | --- |
| Q19 | Compare the environmental impact of informal e-waste recycling versus regulated e-waste recycling facilities. | 10 marks |
| Q20 |  |  |
| Q21 |  |  |
| Q22 |  |  |

Section C

| 23 | A small office has 10 desktop computers, after the installation of additional drives in each computer, the IT manager wants the drives to be organized into separate partitions so that: - Drive C: store the operating system and installed application - Drive D: store user’s documents and office files - Drive E: store backup images and maintenance tools If each drive has ITB. The manager asks you to develop a step-by-step disk partitioning procedure that junior technician can follow safely during system setup. Task: create a step-by-step procedure for partitioning a new hard drive into multiple section for system files(200GB)user data(500GB)and backup storage(300GB).Include preparation, partition creation, formatting and verification steps. | 15marks |
| --- | --- | --- |
| 24 |  |  |